CAPTAIN 6

6-ZONE INTRUDER ALARM SYSTEM

INSTALLATION GUIDE
System ver. 6.XX

Limited Warranty

PIMA Electronic Systems Ltd. does not represent that its Product may not be compromised and/or circumvented, or that the Product will prevent any death, personal and/or bodily injury and/or damage to property resulting from burglary, robbery, fire or otherwise, or that the Product will in all cases provide adequate warning or protection.  The User understands that a properly installed and maintained equipment may only reduce the risk of events such as burglary, robbery, and fire without warning, but it is not insurance or a guarantee that such will not occur or that there will be no death, personal damage and/or damage to property as a result.

PIMA Electronic Systems Ltd. shall have no liability for any death, personal and/or bodily injury and/or damage to property or other loss whether direct, indirect, incidental, consequential or otherwise, based on a claim that the Product failed to function.

Warning: The user should follow the installation and operation instructions and among other things test the Product and the whole system at least once a week.  For various reasons, including, but not limited to, changes in environment conditions, electric or electronic disruptions and tampering, the Product may not perform as expected.  The user is advised to take all necessary precautions for his/her safety and the protection of his/her property.

This document may not be duplicated, circulated, altered, modified, translated, reduced to any form or otherwise changed, unless PIMA’s prior written consent is granted.

All efforts have been made to ensure that the content of this manual is accurate.  Pima retains the right to modify this manual or any part thereof, from time to time, without serving any prior notice of such modification.

Please read this manual in its entirety before attempting to program or operate your system.  Should you misunderstand any part of this manual, please contact the supplier or Technician of this system.

Copyright  2012 by PIMA Electronic Systems LTD. All rights reserved. E&OE.

Default system codes:

Master:       5555

Technician: 1234

Table of Contents

1.	Introduction	5

1.1	Safety Instructions	5

1.2	Version 6.10 updates	5

1.3	Power consumption	5

1.4	Signs in this guide	6

1.5	Main features	6

1.6	The LCD Keypad	7

1.7	The PCB	7

1.7.1	Current limit thermal fuses	8

1.8	Battery jump-start	9

2.	Connecting and Wiring	10

2.1	The system’s BUS	11

2.2	Z1-Z6 zones and power	11

2.2.1	Common zone wiring	11

2.2.2	EOL resistor loops	11

2.3	SMOKE output	11

2.4	PGM output	11

2.5	SIREN output	11

2.6	KEYPAD	12

2.6.1	Setting the keypad’s ID	12

2.7	AUDIO	13

2.7.1	Connecting and programming the MIC-200	13

2.7.2	Connecting and programming the VU-20U	14

2.7.3	Connecting the GSM-200	14

2.7.4	Using the VU-20U with the GSM-200	15

2.7.5	TELEPHONE	15

2.8	TRANSMITTER	16

2.9	JP3: EOL resistor jumper	16

2.10	AC	16

2.11	BATTERY	16

2.12	Connecting key switch and remote control	17

3.	PROgramming Options	17

3.1	Programming via the PRG-22	17

3.2	Programming via the COMAX	18

3.2.1	Locally	18

3.2.2	Remotely	18

3.2.3	Programming via an LCD Keypad	18

3.2.4	Parameters bar	19

3.2.5	Default codes	19

3.3	Initializing the keypad	19

3.4	Setting date & time	20

4.	the technician menu	20

5.	Programming the system	21

5.1	Zones	21

5.1.1	Zone sensitivity	21

5.1.2	Zone characteristics	21

5.1.3	Zone responses	22

5.1.4	Zone name	22

5.2	Monitoring Station communication parameters	23

5.2.1	PSTN & Radio subscriber IDs, Double Report	23

5.2.2	Station format, ACK, Kissoff delay	23

5.2.3	Automatic communication tests	24

5.2.4	GSM-200 cellular module	25

5.2.5	Reporting codes	25

5.2.1	Central Monitoring Station telephones, rings	26

5.3	The SIREN and SMOKE outputs	27

5.4	System configuration	27

5.4.1	Config 1	27

5.4.2	Config 2	28

5.4.3	Config 3	28

5.4.4	Config 4	29

5.4.5	Config 5	30

5.4.6	Config 6	30

5.5	System responses	31

5.5.1	The response to PANIC alarm	31

5.5.1	The response to AC failure	31

5.5.2	The response to battery failure	31

5.5.3	The response to phone line failure	32

5.5.4	The response to zone failure	32

5.5.5	The response to arming/disarming using a key switch	32

5.6	The Entry/Exit delay times	32

5.6.1	User partitions	33

5.7	The Technician code	33

5.8	Defaulting the system & fast loading	34

5.8.1	Defaulting the system	34

5.8.2	Fast parameter loading	34

6.	Troubleshooting	35

6.1	Other faults	36

6.1.1	Radio transmitter	36

6.1.2	Telephone	36

6.2	Retrieving the Master code	36

7.	Partitions	37

8.	Supplemen tary ProducT	38

9.	Appendix – Default system parameters	39

9.1	System	39

9.2	Configuration	39

9.3	Responses	39

9.4	Zone characteristics	40

9.5	Report format and default codes	40

9.6	Users & partitions	41

9.7	Timers	41

9.8	Default codes	41

10.	Appendix B - Formats	42

10.1	Pulse (4-2)	42

10.2	DTMF (4-2)	42

11.	Appendix C – Entering text	43

# Introduction

This guide provides the installation, wiring and programming instructions for PIMA’s CAPTAIN 6 intruder alarm system.

The CAPTAIN 6 has many features that fits customer’s individual needs, and yet it remains easy to install and simple to program and use, both by the end-user and the Technician.

CAPTAIN 6 is secured against Radio-Frequency (RF) and Electro-Magnetic Interferences (EMI).

Information on programming user codes and other end user parameters can be found in the CAPTAIN 6’s User Guide.

## Safety Instructions

Your CAPTAIN 6 Alarm System has been registered in accordance with EN60950 and its rules.

EN 60950 requires us to advise you the following information:

In this alarm system hazards of fire and electric shock exist. To reduce the risk of fire or electric shock, do not expose this alarm system to rain or moisture. Pay attention: Telephone cords could be a good conductor for lightings energy.

Do not open the door of the alarm system. Dangerous high voltages are present inside of the enclosure. Refer servicing to qualified personnel only.

This alarm system should be used with AC 230V/110V, 50Hz, protected by anti electric shock breaker. To prevent electric shocks and fire hazards, do NOT use any other power source.

Do not spill liquid of any kind onto the unit. If liquid is accidentally spilled onto the unit, immediately consult a qualified service.

Install this product in a protected location where no one can trip over any line or power cord. Protect cords from damage or abrasion.

Disconnect all sources of power supply before proceeding with the installation.

Do not install low voltage wires near AC power wires; they should be separated.

Use only standard AC transformer.

Connect the AC transformer output to the terminal block on the control panel as marked.

Connect the AC line cord to line power terminals as marked. (GND; N; L).

## Version 6.10 updates

Battery jump-start. See section ‎1.8.

New PCB version C: siren type jumpers have been cancelled, new EGND terminal was added.

## Power consumption

| Module | Details |
| --- | --- |
| Hunter-Pro PCB | 12VDC 80mA rms |
| Captain 8 PCB | 12VDC 50mA rms |
| LCD keypad | 12VDC 20mA rms |
| LCD keypad illuminating | 12VDC 110mA rms |
| I/O-8N | 12VDC 70mA rms |
| I/O-16N | 12VDC 80mA rms |
| I/O-R | 12VDC 130mA rms |
| EXP-Universal | 12VDC 10mA rms |
| net4pro | 12VDC 100mA rms |
| net4pro-i | 12VDC 70mA rms |
| OUT-1000 | 12VDC 15mA rms |
| IO-WN | 13VDC 100mA rms |
| GSM-200 | 13.8VDC 250mA rms |
| MIC-200 | 12VDC 5mA rms |
| TRU/TRV | 13.8VDC 10mA rms |
| VU-20N/U | 12VDC 45mA rms |
| RXN-400/410 | 13.8VDC 15-20mA rms |
| SMS-100 | 13.8VDC 20mA rms |
| OUT-1000 | 12VDC 15mA rms |
| VVR | 12VDC 360mA rms |

## Signs in this guide

|  | Warning |
| --- | --- |
|  | Note |
|  | Press briefly |
|  | Press and hold a key until confirmation beep is sounded |

## Main features

- 6 alarm zones

- Up to 2 partitions

- Up to 4 Monitoring Station telephone numbers and 3 owner’s

- Various ways for arming and disarming: keypad, key switch, remote control auto-arming

- 2 keypad types: LCD (RXN-400/410), LED (RX-6/406)

- Up to 6 addressable keypads

- 2 operating modes: Full and Home (partial)

- System operations are fully logged, part in a non-volatile memory

- Various codes: Master, 8 Users, Short

- User Code #8 can serve as PANIC alarm

- Temporary zone bypassing

- “Chime” monitoring mode per zone

- Built-in PSTN dialer, Long-range radio and cellular communication

- 2 key combinations for DURESS and FIRE codes

## The LCD Keypad

LCD keypad

The CAPTAIN 6 is fully controlled by the LCD keypads.

The keypads number buttons are used for accessing the user and technician menus and for programming.

The 4 buttons on the right are used mostly for programming and menu navigation.

The LCD screen is made of 2 character lines.

## The PCB

CAPTAIN 6 PCB

| No. | Terminal/ Connector | Description |
| --- | --- | --- |
|  | Z1-Z6 | 6 inputs for dry contact detectors & (-) terminal |
|  | (+)/(-) | Power supply for PIR/ultrasonic/etc. detectors. |
|  | SMOKE | Voltage output for resetting Smoke & Fire detectors. The output is temporarily switched to (-) when triggered. See section ‎5.3. |
|  | PGM | Auxiliary output. When triggered, it can be disconnected or switched to GND. See sections ‎5.4.3, ‎5.4.4.1. The output can be used to trigger the MIC-200 microphone and the VU-20 voice module. |
|  | SRN +/- | Siren Output 2 sirens can be connected to this output in parallel. See section ‎2.5. |
|  | KEYPAD | Terminals for connecting the system BUS. The terminals are: (+V)/ (-): power supply IN/OUT: DATA Up to 6 keypads can be connected to the system All PIMA LED keypads can be connected to the system Do not connect anything but keypads to the keypad power supply!. |
|  | AUDIO (-) /IN/OUT | - Terminals for connecting the MIC-200 microphone and the VU-20 voice module. See sections ‎2.6.1, ‎2.7.2, ‎5.1.3. |
|  | TELEPHONE LINE | - Line-in terminals |
|  | TELEPHONE SET | - Line-out terminals for connecting phone sets, fax, etc. |
|  | EGND | Earth Ground terminal. Can be used in areas of severe electrical activity (abnormal levels of lightning or electrical discharge). When using PIMA’s integrated transformer, earth ground is not required. Only when using external transformer and lightning conditions are severe, the EGND terminal can be used. Connect the terminal to earth grounds, such as metal cold water pipe or AC power outlet ground |
|  | KEYPAD | - A Molex connector for quick connection of the technician keypad |
|  | Transmitter | - A connector to the TRV/TRU-100 long range radio transmitters and the GSM-200 cellular module |
|  | Backup Battery cables | RED cable: (+) BLACK cable: (-) Inverting the battery contacts can damage the PCB. |
|  | AC | 14-16 VAC Voltage input The cross-section area of the AC cable must be at least 0.75mm2 |
|  | JP3 | - EOL resistor loops jumper. See section ‎2.9 |

### Current limit thermal fuses

| Fuse | Details |
| --- | --- |
| F1 | 0.9A, siren power supply |
| F2 | 750mA, Keypads and detectors power supply |
| F3 | Battery |

## Battery jump-start

Starting PCB version 3610105 Rev. C., during AC fault, if the backup Battery’s voltage drops under 8V, the control panel disconnects it to prevent full battery discharge. This feature extends the battery life cycle.

Because of this, the control panel cannot be powered up using only the battery, and must be connected to AC voltage first.

When AC voltage is not available, do the following:

  - Connect the control panel to the Battery.

  - Briefly short the Battery’s (-) terminal to the control panel’s (-) terminal. See the diagram.

  - The control panel will now power up.

# Connecting and Wiring

CAPTAIN 6 wiring diagram

## The system’s BUS

The system’s BUS is made of 4 wires: 2 for Power - (+)/(-) and 2 for DATA - (IN)/(OUT).

The BUS uses PIMA proprietary protocol.

The overall length of the BUS cannot exceed 500 meters. Call PIMA support when a longer distance is required.

## Z1-Z6 zones and power

### Common zone wiring

Optional zone wiring

### EOL resistor loops

To set a zone as an EOL resistor loop, see the “Zone Characteristics” menu, section ‎5.1.2.

Set the number of EOL resistors (one or two) per system, in the “Config. 5” menu. See section ‎5.4.4.1.

## SMOKE output

Connect smoke or fire detectors between this terminal and (-).

When the alarm is set off, the output is disconnected for a minute to reset the detector.

To manually reset the output, 

## PGM output

Connect devices that require a trigger to operate, between this terminal and (-).

Can also be used for indicating on alarms and faults with lamps, vocal modules, etc.

## SIREN output

Connect the siren/horn to the SRN (+)/(-) terminals. See the next diagram.

The siren’s type is determined by the siren tone. See section ‎5.3.

A second siren can be connected in parallel. See the next diagram.

Siren wiring

## KEYPAD

Keypads are connected to the control panel’s KEYPAD terminals over the BUS. See the next figure and table.

| Keypad wiring |  | Keypad; Control Panel. 1 -; 1 -. 2 +; + 2. 3 OUT; 3 IN. 4 IN; 4 OUT. |
| --- | --- | --- |
|  | The keypads power should only be used for keypads. The BUS wires should not be passed too close to telephone wires. |  |

### Setting the keypad’s ID

- Open the keypad’s back cover;

- Short JP1’s pins 1 & 2; the message onscreen should say: “Enter new ID:  X”. See the next diagram.

- Enter the new ID - 1 to 8;

- Short JP1’s pins 2 & 3;

- Repeat the process for the other keypads. Note, that each supervised keypad must have a unique ID and that the IDs must be set in ascending order.

- If keypad supervision is not required, the keypads’ IDs should be set to zero.

An LCD keypad PCB

## AUDIO

### Connecting and programming the MIC-200

MIC-200 Wiring

The MIC-200 is a microphone that is used for listen-in, when the alarm is set off.

To program a zone to activate the MIC-200, follow the next steps:

In the “Zone responses” screen (see section ‎5.1.2), go to the zone that should activate the MIC-200 and set “G” (PGM) to “+”. This will set the zone to trigger the PGM output, to which the MIC-200 is connected, when the zone is triggered.

In the “Config 3” (see section ‎5.4.3) set “G” (delayed PGM) to “+”. This will delay the triggering of the PGM output, so the dialer can call the subscriber first.

### Connecting and programming the VU-20U

| Vu-20U Wiring |  |
| --- | --- |

The VU-20U is a voice module that can play up to 2 recorded messages over the phone, when the alarm is set off.

The messages are played instead of the alarm siren sound and can contain the zone name where the alarm was set off, for example.

To program a zone to activate the VU-20U, follow the next steps:

In the “Zone responses” screen (see section ‎5.1.2), go to the zone that should activate the VU-20N and set “G” (PGM) to “+”. This will set the zone to trigger the PGM output, to which the VU-20N is connected, when the zone is triggered.

In the “Config 3” (see section ‎5.4.3) set “V” (Voice unit) and “G” (delayed PGM) to “+”1. This will cause the control panel to play the recorded messages and to delay the triggering of the PGM output, so the dialer can call the subscriber first.

### Connecting the GSM-200

GSM-200 wiring

The GSM-200 is PIMA’s cellular module.

To connect GSM-200:

Connect the GSM-200 YELLOW wire to AUDIO IN;

Connect the ORANGE wire to AUDIO OUT;

Connect the serial wire braid between the module and the TRANSMITTER terminal on the control panel.

### Using the VU-20U with the GSM-200

To use both the VU-20U and the GSM-200, you must connect a 5.1 k resistor to the VU-20U blue wire, near the AUDIO IN terminal. See the next diagram.

VU-20U with GSM-200 resistor diagram

### TELEPHONE

Telephone wiring

Connect the telephone line wires TELEPHONE LINE terminals.

Use the SET terminals to connect other telephone sets, fax or answering machine to the SET terminals.

This setup enables the control panel to disconnect the SET terminals and make a phone call, when an alarm is set off or a fault occurs.

#### The system dialer

The CAPTAIN 6 includes a dialer for calling the end user and the Central Monitoring Station phone numbers, via PSTN or GSM networks.

The dialer calls each end user number twice and sounds a synthesized alarm tone or voice message (if a VU-20 voice unit is installed).

The dialer terminates the calling cycle in the following circumstances:

- The system is disarmed;

- All calls were completed (2 calls per number).

## TRANSMITTER

Radio transmitter wiring

Connect the TRV-100 or TRU-100 long range radio transmitters, to the TRANSMITTER terminal on the control panel.

If the transmitter uses the second channel, connect its wires to the triggering output, e.g., PGM output.

## JP3: EOL resistor jumper

Set the JP3 jumper, according to the EOL resistor loops. See the next table.

| Resistors (KΩ) | Short pins |
| --- | --- |
| 10, 13 (default) |  |
| 10, 10 |  |
| 5.1, 6.8 | No jumper |

## AC

The cross-section area of the AC wires must be at least 0.75 mm2.

The hole through which the main cable passes must have either a grommet or bushing.

The wires of the cable must be tied together with a cable tie. The flammability of the cable tie must be UL 94 V-2 or better.

Connect the wires to the AC terminals of the transformer housing.

Use an Ohmmeter and check the continuity between the grounding points on the PCB and the grounding of the electricity outlet. The resistance must be less than 1.

|  | The system should be connected to an automatic circuit breaker. If Earth Grounding is required, connect it to the EGND terminal on the PCB. See section ‎1.7. |
| --- | --- |

## BATTERY

Connect a rechargeable Lid-acid 12V battery.

The charging voltage is 13.8VDC.

The system performs a load battery test each time:

Every time it is armed;

Every 3 minutes and every 4 hours;

Upon connecting to power

To manually test the battery -

If the test fails, the system will respond as programmed in the “System responses” screen - sounding the siren, calling the Monitoring Station, etc. See section ‎5.1.3.

## Connecting key switch and remote control

Connect the switch or the remote control between zone #6 and (-).

Set zone #6 as normally open and as Key input (see section ‎5.4.3).

Set the key as momentary or latch type (see section ‎5.4.3).

For better protection it is recommended to connect a 10K EOL resistor.

Key switch wiring

# PROgramming Options

There are 3 ways to program the system:

- Locally, with the PIMA Fast programmer PRG-22. The programmer can store up to 4 predefined parameter sets.

Locally or remotely (via telephone or the GSM-200 DATA channel), using PIMA’s COMAX Upload/Download application.

Via an LCD keypad.

## Programming via the PRG-22

The PRG-22 is a memory card used for storing parameter sets for fast downloading and quick installation.

The sets are uploaded by the COMAX application.

The PRG-22 can store up to 4 different presets.

It connects to LCD keypads only. Do not connect the PRG-22 to a LED keypad. It may damage the programmer.

For downloading instructions, see section ‎5.8.2.

Connecting PRG-22 to LCD Keypad

## Programming via the COMAX

### Locally

Connect the PC/laptop with the COMAX upload/download application to an LCD keypad, using the LCL-11A adaptor. See the diagram.

Use a Serial-to-USB adaptor to connect to a USB socket.

Connecting a PC with the COMAX via LCL-11A to an LCD keypad

### Remotely

The CAPTAIN 6 can be programmed remotely via the telephone using the COMAX.

Refer to the COMAX user guide for detailed information.

### Programming via an LCD Keypad

LCD screen

The most common way to program the CAPTAIN 6 is via an LCD keypad.

The system has two menus: User and Technician, each with its own screens:

The User menu is used to program parameters such as time, date and user codes. To access it, enter the MASTER code.

The Technician menu contains all the technical and the reporting parameters.

In both the User and Technician menus, the various screens are accessed by pressing the number keys. Some menus have sub-menus.

Press / to scroll between the number menus and between parameters.

Press  to access the menu, and again, to access sub-menus and store parameters.

Press  to discard changes and exit the menus.

### Parameters bar

In some screens, multiple parameters appear as an options bar.

The parameters, represented by letters, are either marked with “+”, i.e. are enabled, or marked with “-”, i.e. are disabled. For example:

A short description of the parameter is displayed briefly on the right side, as the curser is moved. In the previous example, the parameters are zone #1 characters and the current parameter is Bypass.

Press / to scroll between the parameters on the same screen.

Press  (toggle mode) to set as “+” (enable) or “-” (disable).

To save, press

In the zone characters screens, press a number between 1-6 to display the desired zone, or press  to display the next zone without saving.

### Default codes

CAPTAIN 6 factory default codes are as follows:

Master: 5555

Technician: 1234

## Initializing the keypad

Upon connecting the keypad to power a long tone is sounded, followed by the next initialization screens:

The first screen displays the keypad version and ID.

The third is the system’s main screen. It shows the hour and the zones’ status indicators.

The word “Clock” and the flashing hour indicate that the time is not set and that it should be (see section ‎3.4). Because of that, the RED Fault LED will be flashing. Other faults, if exist, will also be displayed.

## Setting date & time

 MASTER code      current time (HH:MM)     current date (YY:MM:DD)   

The system time must be accurately set for the system to log the events and report them with a time stamp.

Use / to correct data, or press  and start again.

# the technician menu

To access the Technician menu:  Technician code  - OR -

 MASTER code    Technician code

The following are the keypad’s keys and their function screens.

| Key | Function |  | Page |  | Key | Function |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Zone sensitivity |  | 21 |  |  | System responses |  | 31 |
|  | Zone characteristics |  | 21 |  |  | Delay times |  | 32 |
|  | Zone responses |  | 22 |  |  |  | User partitions | 33 |
|  |  | Zone name | 22 |  |  | Technician code |  | 33 |
|  | Subscriber ID |  | ‎5.2.123 |  |  | System defaults |  | 34 |
|  |  | GSM Unit | 25 |  |  |  | Fast load (1-4) | 34 |
|  | Report codes (PSTN) |  | 25 |  |  |  |  |  |
|  |  | Report codes (Radio) | 25 |  |  |  |  |  |
|  | Monitoring Station phone numbers |  | 26 |  |  |  |  |  |
|  |  | Phone line | 27 |  |  |  |  |  |
|  | Siren and SMOKE outputs |  | 27 |  |  |  |  |  |
|  | System configuration (6 screens) |  | 27 |  |  |  |  |  |
|  |  | Config 1 | 27 |  |  |  |  |  |
|  |  | Config 2 | 28 |  |  |  |  |  |
|  |  | Config 3 | 28 |  |  |  |  |  |
|  |  | Config 4 | 29 |  |  |  |  |  |
|  |  | Config 5 | 30 |  |  |  |  |  |
|  |  | Config 6 | 30 |  |  |  |  |  |

# Programming the system

## Zones

### Zone sensitivity

 Technician code          zone #1 sensitivity OR   to set zone #2 sensitivity OR   to the next zones.

Zone sensitivity is the time in milliseconds a zone must be triggered, before it can trigger the alarm. If the zone is opened for a shorter time, the control panel ignores that.

Zone sensitivity is set per zone.

The sensitivity is set in a number that is multiplied by 50. For example, entering “8” means a sensitivity of 8 times 50, that is 400 milliseconds (0.4 seconds).

The minimum triggering time is 50 milliseconds and the maximum is 12,500 (12.5 seconds). In the values that can be set, the minimum is “1” and the maximum is “250”.

The zone sensitivity is set in increments of 50 milliseconds (0.05 seconds).

### Zone characteristics

 Technician code         set the characteristics of zone #1 

Set the characteristics of each zone: delay, N.O./N.C., etc.

The first screen is zone #1 screen. You can enter any other zone number and program its characteristics. (See how in section ‎3.2.3).

To confirm and progress to the next zone, 

The characteristics are set per zone independently.

To progress to the next zone without saving, 

The following is a description of the zone characteristics:

| Par. | Onscreen | Characteristic description |
| --- | --- | --- |
| B | Bypass | The zone is permanently bypassed and will not trigger the alarm at any time, if violated. |
| O | N.O. | “Normally Open” zone |
| F | 24 Hours | 24-hour zone. The zone will trigger the alarm at any time, if violated, regardless of system state: Armed, Disarmed, or Home |
| H | Active in “Home” mode | The zone will be armed when the “Home” mode is armed. |
| I | Entry delay | Triggering this zone will be delayed to the extent of the Exit or Entry delay, before it triggers the alarm. |
| D | Entry follower | If triggered, as long as any delayed (Entry & Exit) zone is open, this zone will not trigger the alarm if. |
| E | End-Of-Line | This zone is connected over EOL resistor loop. |

### Zone responses

 Technician code        set the responses of zone #1 

Set the responses of each zone when it is violated.

The first screen is zone #1 screen. You can enter any other zone number and program its responses. (See how in section ‎3.2.3).

To confirm and progress to the next zone, 

The responses are set per zone independently.

To progress to the next zone without saving, 

If both partitions #1 and #2 are active, the programmed zone will be triggered only when both partitions are armed.

The following table describes the zone responses:

| Par. | Onscreen | Response description |
| --- | --- | --- |
| S | Siren | Trigger the siren/s |
| P | Communic. | Report the Central Monitoring Station and the owner over the phone (PSTN and/or GSM according to programming). |
| G | PGM | Trigger the PGM output |
| F | Fire | Reset the SMOKE output. The GND connection of the output will be disconnected for one minute. |
| 1 | Partition #1 | “+”: allocate the zone to partition #1 |
| 2 | Partition #2 | “+”: allocate the zone to partition #2 |

### Zone name

 Technician code            the zone name 

A zone name can have up to 8 characters.

See section ‎10 for instructions on how to enter text in CAPTAIN 6.

## Monitoring Station communication parameters

 Technician code                                         

### PSTN & Radio subscriber IDs, Double Report

 Technician code         “Ph” - set the PSTN subscriber ID for Monitoring Station #1/Partition #1   “Rad” - set the Radio subscriber ID      to exit

-OR-

 set the same parameters for Monitoring Station #2 (“Double Report”) or Partition #2 

In CAPTAIN 6, you can program a separate subscriber ID for the PSTN and the Radio reports.

A subscriber ID can have up to 4 digits.

The “Double Report/Partition” screen enables you to program a second ID if the control panel reports to a second Central Monitoring Station, or if Partition #2 is in use. The options of using the second IDs are listed in the next table.

|  | I.D=0 is as if no subscriber number is programmed The largest subscriber number for the PIMA format is 8000. |
| --- | --- |

### Station format, ACK, Kissoff delay

 Technician code        X3    the station’s PSTN & Radio formats     
set the station’s ACK timeout (in seconds) and the Radio re-transmissions   set the Radio’s Kissoff delay (in 250ms increments per number)     set the number of frames per transmission (min-max) 

#### Station format

The Central Monitoring Station format in CAPTAIN 6 is set by 3 numbers: 2 for the PSTN format and one for the Radio station number.

In the default screen “0 230” stands for the Ademco® ContactID protocol. See the full format list in “Appendix B”, from page 42.

Parameter ”T” in this screen stands for the Radio station number, given by the Monitoring Station.

#### ACK waiting time & Radio re-transmissions

Set the ACK waiting time. Minimum waiting time: 20 seconds. Maximum: 60.

The ACK is a signal sent from the Monitoring Station, indicating that the data has been correctly received.

Set the number of the long-range Radio re-transmissions per event. The default (and minimum) number is 5. The duration between the transmissions is 10 seconds, and cannot be changed.

#### Kissoff delay

Set the delay time for the Kissoff tone, before a report is sent.

The Kissoff tone indicates that the message has been received successfully in the Monitoring Station.

The delay is set in 250msec increments. For example, setting the delay to 8 means, 8X250=2000msec (2 sec.).

#### Frames per transmission

Set the number of frames per Radio transmission. Min-max

### Automatic communication tests

 Technician code        X7   set the daily time for sending test reports in HH:MM format     set an interval for sending test reports in HH:MM format 

Set the daily time for the alarm system to perform an automatic communication test with the Monitoring Station.

The time is set in HH:MM format.

If the test fails, a report is generated.

#### Auto test interval

Set an interval for the alarm system to perform to perform an automatic communication test with the Monitoring Station.

The time is set in HH:MM format, i.e., every X hours and Y minutes.

|  | You can set an interval as well as daily tests. |
| --- | --- |

### GSM-200 cellular module

 Technician code        X9         set the module’s parameters     set a prefix number where required 

By default, the GSM module uses the same account ID as the Radio channel.

If the GSM-200 is installed as a sole dialer, verify that “P - Telephone” in the “Config 1” screen is set as “-” (see section ‎5.4).

For information on how to install the GSM-200, turn to the GSM-200 Installation guide.

To GSM-200 parameters are:

| Par. | Onscreen | When settings to “+/-”... |
| --- | --- | --- |
| G | GSM Exist | The GSM-200 is installed |
| E | Open Rep. Imd | “+”: Arming/Disarming reports will be sent via the GSM module after one PSTN failure “-”: Arming/Disarming reports will be sent via the GSM module only after 4 successive PSTN failures Regardless of parameter “E”, when the alarm system fails to report via the PSTN channel, all the events are immediately transmitted via the GSM one, until the PSTN channel is reinstated.. |
| R | Radio Account | “+”: The GSM reports will be sent under the Radio account I.D. “-”: The GSM reports will be sent under the PSTN account I.D. |
| T | Automatic Test | Automatic test reports will be sent via the GSM module too |
| P | Parallel (report) | All PSTN reports will be sent via the GSM module too |

#### GSM pre number

Set a pre number (prefix) if the GSM module dials over a switchboard or phone system, or as an area code.

### Reporting codes

 Technician code        ...   ...                                 ...     ... the same screens.

Set the telephone <P> and Radio <T> reporting codes. The codes are the same.

The default “FF” codes are to be used with the following formats: ContactID, PAF, NPAF. All other formats require specific other than “FF” codes.

A code is made of one or two digits. Each digit can take on the values 0 through 15, whereby the values of 10 through 15 are represented by the letters A-F as follows: A-10, B-11, C-12, D-13, E-14, F-15

These values are entered using, as follows:

- To obtain a digit between 0-9, press on the corresponding key directly.

To obtain a letter between A and F, press  until you obtain the desired letter.

The default codes are as follows:

| Code | Report |
| --- | --- |
| Z1, Z2... Z6 | Alarm generated in zones 1 - 6. |
| RS1, RS2... RS6 | Restore report for zones 1 - 6. The report is sent at the end of the siren time, or when the system is disarmed immediately after alarm. |
| Zone Fail | Zone fail |
| AC/RESTR | AC fault/Restore |
| LB/RESTR | Low battery/Restore |
| PF/RESTR | Low PCB voltage (less than 9V)/Restore This code usually indicates that the battery is low and must be replaced immediately.. |
| PH/RESTR | Telephone fault/ Restore |
| TST | Test report (automatic/manual) |
| PNC | Panic code pressed + |
| FUS/RESTR | Zone power fault/Restore |
| ONG/OFG | System was armed/disarmed with a code other than a user code |
| ON1-8 OFF1-8 | System was armed/disarmed using user code 1-8 |
| BPS/RST | System was armed with bypassed zones |

### Central Monitoring Station telephones, rings

 Technician code         enter the first number   enter the other numbers    set a prefix if necessary        set the rings number 

Set up to 4 telephone numbers of the Central Monitoring Station.

The dialer attempts to call the numbers in the order as programmed, 2 trials per number, 8 altogether. If all trials fail, a communication fault report is generated and the Red FAULT Led will be flashing.

When dialing through a switchboard, press  after the access digit (e.g. 8) for a 1 second delay.

#### Pre-number

Set a pre number (prefix) if necessary.

Setting a pre number increases the number of digits available for the Monitoring Station telephone numbers from 16 to 22.

#### Number of rings

The number of rings before the control panel pick up a call is set for the COMAX upload/download software.

Usually, there is no need to change the default number of rings (10).

## The SIREN and SMOKE outputs

 Technician code         set the SIREN output tripping time (in sec.)    set the SMOKE output disconnection time (Min.=60? in sec.)    set the siren tone (0-9) 

Set the SIREN and SMOKE outputs timers.

The SMOKE output timer is the time the output is disconnected for the detector to be reset.

The siren’s tones 0-8 are variable tones for DC sirens.

Set to tone #9 when using a self-driven siren.

## System configuration

The System configuration is a set of 8 consecutive screens that includes various parameters for configuring the CAPTAIN 6 alarm system.

### Config 1

 Technician code             set the parameters 

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| P | Phone | The control panel is connected to telephone line. |
| P | TrgTst-P | While the system is armed, the control panel will send a TEST report by the phone, in response to 2 phone rings. |
| T | TrgTst-T | While the system is armed, the control panel will send a TEST report by the Radio, in response to 2 phone rings. |
| P | AutTst-P | The system will send a periodic TEST report by the phone. See section ‎5.2.3.1 |
| T | AutTst-T | The system will send a periodic TEST report by the Radio. See section ‎5.2.3.1 |
| B | AutoBYpS | A zone that triggers the alarm 3 consecutive times will automatically be bypassed until the system is disarmed. |

### Config 2

 Technician code         X2    set the parameters 

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| T | DialTnBp | The system will not wait for a dial tone before dialing. This is useful for “noisy” telephone lines. |
| D | Dlayed On | Arming the system will be delayed until a confirmation message is received from the Monitoring Station. The message “Message received” will then be displayed on the LCD keypads. |
| Z | Zone Disp | While the system is armed, LCD keypads will display the zone status. LED keypads display open zones regardless of the system state. |
| B | BZR-ALARM | The keypad’s Chime will indicate when the alarm is triggered. |
| R | RST BY ZN | “+”: Zone restore immediately after alarm is reported. “-”: The control panel will send Restore report after siren time ends. Do not enable this parameter when using the PAF format.. |
| T | Tone Dial | “+” The control panel will use tone dialing. “-” The control panel will use pulse dialing. |

### Config 3

 Technician code         X3:   set the parameters 

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| S | Not in use | - |
| D | Dnld. Disa | By default, remote connection is disabled (the parameter is set to “+”) To enable it: - Set the parameter to “-”. - On the first time remote access is required, when calling the system, the MASTER code must be entered, then the key must be pressed twice. - Without enabling this parameter, this process allows 4 minutes for the Technician to connect and Upload/Download data. |
| V | VoiceUnit | A VU-20 voice module is connected to the system. Instead of sounding the synthesized alarm tone when calling the customer, a recorded message will be played. In addition to enabling this parameter, set to “+” “G - Del. PGM” in this screen and “G - PGM” in the “Zone responses” screen (see section ‎5.1.3). |
| K | Zone6=key | Zone #6 is set as KEY switch input. |
| K | Key=State | “+”: The KEY switch is set as 2-state key (ON/OFF, toggle). “-”: The KEY switch is set as momentary/wireless remote key. |
| G | Del. PGM | Delayed PGM: the PGM output will be switched to “-” only after the system completes dialing, so that the voice module recorded message will not be played before the call is answered. |

### Config 4

 Technician code         X4   set the parameters 

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| L | Line Check | The system will perform periodic telephone line checks when it is armed |
| L | Chk in off | The system will perform periodic telephone line checks when it is disarmed |
| D | Dbl reprt | “Double report” parameter. See the following section |
| A | Reprt all | “Double report” parameter. See the following section |
| S | Smke inv. | The smoke output will normally be disconnected, and switched to GND when the alarm is set off. To be used when using piezoelectric sirens or when the SMOKE output follows the siren state. To set the output’s trip time, see section ‎5.3. |
| S | 2 rng snp | 2-ring line snapping (“Answering machine override”) is enabled. The process of calling the control panel is as follows: - Call the system, wait for 2 rings and hang up; Wait 10 seconds; Call the system again; The system will pick up the call immediately. |

#### Double report parameters (D/A)

The next table describes to two “Double report” parameters taken from the “Config 4” previous screen.

When “Double report” is enabled, telephone numbers #1 & #2 of the Monitoring Station will be of station #1, and numbers #3 and #4 will be of station #2 (see section ‎5.2.1).

| D | A | Description |
| --- | --- | --- |
| - | - | No “Double report” |
| + | - | “Double report” as follows: - All events are reported to Monitoring Station #1; - Monitoring Station #2 is reported only on arming, disarming, faults and tests. |
| + | + | “Double report” is fully enabled: all events are reported to Monitoring Station #1 & #2 |

### Config 5

 Technician code         X5   set the parameters 

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| B | ByPsReprt | Bypassing a zone will be reported to the Monitoring Station |
| K | RX130Kbrd | The RX-130 keypad is in use. |
| B | ByPsRqest | The system cannot be armed if any zone is open, including “Delayed” and “Entry Followers” zones. |
| 2 | EOL 2 Res | All EOL resistor loops have 2 resistors and can indicate on cut and short |
| L | Listen-in | A MIC-200 microphone is connected to the control panel. The microphone will be triggered via the PGM or SMOKE outputs (see section ‎5.3). The trigger for the listen-in is 3 minutes long. Note: Relevant only for ContactID |
| S | Del.Smoke | SMOKE output delay. See section ‎5.3 |
| 4 | RXN-406 | The RXN-406 or RX-6 keypads are in use. |

### Config 6

 Technician code         X6    set the parameters 

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| P | 8=Panic | Entering User code #8 will send a PANIC alarm |
| D | Arm.Dis.fail | The system cannot be armed with AC, low battery or phone fault |
| X | Not Used | - |

## System responses

 Technician code                               

Set the responses to some system events and faults.

Each parameter screen has 4 or 5 parameters, each one stands for a potential response to the event that is displayed on the same screen, e.g., “zone fault”.

In the response time screens, set a delay for the control panel to report the on event. A zero time is no delay.

### The response to PANIC alarm

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| S | Siren | The siren will be triggered |
| P | Communic. | The alarm will be reported to the Monitoring Station and the end user via the phone, and to the Monitoring Station only via the radio. |
| G | PGM | The PGM output will be triggered |
| F | FIRE | The SMOKE output will be triggered |

### The response to AC failure

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| S | Siren | The siren will be triggered |
| P | Communic. | The alarm will be reported to the Monitoring Station and the end user via the phone, and to the Monitoring Station via the Radio too. |
| B | Buzzer | “+” keypad buzzer activated |
| G | PGM | The PGM output will be triggered |

#### Response delay time

Set the AC failure report delay time in minutes (up to 250).

### The response to battery failure

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| S | Siren | The siren will be triggered |
| P | Communic. | The alarm will be reported to the Monitoring Station and the end user via the phone, and to the Monitoring Station only via the radio. |
| B | Buzzer | The keypad chime will be activated |
| G | PGM | The PGM output will be triggered |

### The response to phone line failure

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| S | Siren | The siren will be triggered |
| P | Communic. | The alarm will be reported to the Monitoring Station and the end user via the phone, and to the Monitoring Station only via the radio. |
| B | Buzzer | The keypad chime will be activated |
| G | PGM | The PGM output will be triggered |

#### Response delay time

Set the phone line failure report delay time in minutes (up to 250).

### The response to zone failure

Zone failure can occur only in EOL resistor loops.

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| S | Siren | The siren will be triggered |
| P | Communic. | The alarm will be reported to the Monitoring Station and the end user via the phone, and to the Monitoring Station only via the radio. |
| B | Buzzer | The keypad chime will be activated |
| G | PGM | The PGM output will be triggered |

### The response to arming/disarming using a key switch

See section ‎3.2.4 for programming instructions of the parameters bar.

| Par. | Onscreen | When setting to “+” |
| --- | --- | --- |
| S | SirenCheck | The siren will indicate by sounding a beep |
| T | Cntr. Tx | The event will be reported to Monitoring Station by the Radio |
| P | Cntr. Ph | The event will be reported to Monitoring Station by the phone |
| G | PGM | The PGM output will be triggered |

## The Entry/Exit delay times

 Technician code            set the entry delay in seconds (max. 250)      set the exit delay in seconds (max. 250) 

The exit and entry delays refer to the delayed zones, set in the “Zone characteristics” screen (see section ‎5.1.2).

A delayed zone will not sound the alarm if triggered within the delay time.

### User partitions

 Technician code        X3      allocate (set to “+”) users 1-8 to partition #1     allocate (set to “+”) users 1-8 to partition #2 

In this screen you assign users to partitions #1 and #2.

See section ‎3.2.4 for programming instructions of the partitions bar.

A partition is made of several zones.

Using partition allows the end user to arm part of the premises, while other parts are not. In this way, only the armed zones will trigger the alarm when violated; the other zones can be occupied at the same time.

There can be up to 2 partitions in CAPTAIN 6.

A user can arm/disarm only the partition it is allocated to (but can be allocated to both partitions).

|  | When one of the partitions is armed, entering the Master code and pressing will arm the other partition. When both partitions are armed, entering the Master code and pressing will disarm them both. |
| --- | --- |

## The Technician code

 Technician code          enter a new (4-6 digit) Technician code 

The technician code is displayed with asterisk and cannot be revealed otherwise.

If the code is lost, disconnect the control panel from any power for 10 seconds and then re-connect it. For 30 seconds only, the system default codes (Master - 5555, Technician - 1234) will be active.

If the lost code begins with zero, it cannot be defaulted and you should call the support team.

## Defaulting the system & fast loading

### Defaulting the system

 Technician code         to cancel the action  to proceed  ...

The CAPTAIN 6 allows restoring of all system parameters to their factory defaults (except for the Master and Technician codes).

Defaulting erases all previous programming: memory, user codes, zone names, etc.

See “Appendix – Default system parameters” on page 39 for the factory defaults.

### Fast parameter loading

 Technician code         the program number to be downloaded ///. Wait for the process to end  

This screen is used for downloading parameters to the control panel via the PRG-22 programmer.

The parameters are uploaded to the programmer by the COMAX upload/download software.

To start the downloading, connect the PRG-22 programmer to an LCD keypad. See section ‎3.1 for instructions.

# Troubleshooting

When a fault occurs, the Red “FAULT” LED flashes, the Chime sounds beeps and a short description of the fault is displayed.

The following are the faults and their troubleshooting:

| Fault | Description & troubleshooting |
| --- | --- |
| Clock | Appears after power up. Set the time. See section ‎3.4. |
| KEYBOARD NOT CONNECTED | Keypad-control panel communication error. Check/do the following: - The “KEYPAD OUT” wire is connected; The Keypad’s voltage supply is not lower than 13V. If it is, verify that no more than 6 keypads are connected to the panel; Replace the Keypad; Replace the PCB; |
| Battery | Indicates low battery power and appears after battery test and after a prolonged AC failure. Wait 24 hours for the battery to recharge. If the fault persists, replace the battery. |
| Low DC | Low DC supply to the PCB (<9V). Occurs after a prolonged AC failure and before the batter is disconnected to protect it from discharging. Resume AC power. Replace battery if necessary. |
| DC FUSE | Detectors power failure |
| AC Line | AC power failure. Check the AC fuse (F4). |
| Trouble | EOL loop zone failure (short or cut). The letter “F” will be displayed above the troubled zone number. |
| Communic. | Monitoring Station-control panel communication error. Check the following: - The telephone line is properly connected; Parameter “P - Telephone” in “Config 1” screen (see section ‎5.4.1) is enabled (set as “+”); A correct telephone number is programmed for the Monitoring Station. See section ‎5.2.1; The PSTN ID is set (is not zero). See section ‎5.2.1; The programmed Format is the same as the Monitoring Station one. See section ‎5.2.2.1; The dial method - pulse or tone. See section ‎5.2.1; A prefix is programmed if required. See section ‎5.2.1.1. |
| Phone | Phone line failure This fault occurs if there was no dial tone the last time the system checked it. When the system is disarmed, unless the line is re-checked (parameter “Chk in off” in the “Config 4” screen is enabled. See section ‎5.4.4), it will continue to be displayed even after the line returns, until the system is disarmed again. |
| GSM Unit | The GSM-200 module is either not connected or faulty. |

## Other faults

### Radio transmitter

Check the following:

- Proper connection between the control panel and the transmitter;

The Radio ID is set (is not zero). See section ‎5.2.1;

The radio station number is correct; See parameter “T”, in section ‎5.2.2;

The Format is compatible with the one used in the Monitoring Station. See section ‎5.2.2.1.

### Telephone

Check the following:

- The telephone line is properly connected to the IN inputs on the terminal block

In “Config 1” the P for telephone is programmed with “+”

At least one telephone number is programmed for private numbers

Correct telephone numbers have been entered

Correct dial method (pulse or tone)

A prefix (usually 9) is programmed if the system works through a switchboard

## Retrieving the Master code

To assure maximum protection and security, it is not possible to program the system without entering the Master code.  When the Master code is not known, however (if it is forgotten, for example), the following operations must be performed:

- Disconnect mains AC voltage from the system

- Open the control unit box

- Disconnect the battery

- Wait several seconds, connect the battery

- Wait until the display with the clock appears on the keypad

- Enter the factory default Master code (5555)

- Program a new Master code (it is not possible to see the old code)

- Close the control unit box

- Connect mains AC voltage

Note: After battery connection the system enables access by means of the factory code for 30 seconds only. If you do not gain entry in this time, repeat the operations from the beginning.
The above procedure is to be followed also if the technician code has been forgotten (unless the technician code was programmed to begin with digit 0)

System is not answering telephone calls

Check the following:

- The telephone line is properly connected to the IN inputs on the terminal block

- In “Config 1” the P for telephone is programmed with “+”

- The system is programmed to pick up after a reasonable number of rings – Not more than 10

Automatic arming is not functioning

Check the following:

- Clock fault (clock is set to the correct time)

- The Automatic arming feature is activated. See user’s manual.

Activation of a zone does not cause an alarm

Check the following:

- The Zone is not temporarily or permanently bypassed

- The zone is programmed for the correct response (siren, relay, etc.)

- Power supply is not low – AC or correct battery backup power supply

- Detectors are installed correctly and not malfunctioning

# Partitions

CAPTAIN 6 can be configured into 2 partitions with the following settings:

Each zone and each user can be assigned to one of the two partitions or both

Each partition can have a different subscriber (account) ID

All keypads connected to the system show the same display, regardless their partition.

For additional information:

To assign zones to partitions: see section ‎5.1.3

To program subscriber (account) ID: see section ‎5.2.1

To assign users to partitions: see section ‎5.6.1

See “CAPTAIN 6 User Guide” for information on keypads display and partitions

# Supplemen tary ProducT

LCD Keypads

RXN-400 - Small LCD screen

RXN-410 - Large LCD screen

Communication Modules

GSM-200 - GSM/GPRS Transmitter

net4pro - Network Interface Card

TRV-100 - VHF Radio Transmitter

TRU-100 - UHF Radio Transmitter

Wireless Accessories

MCT-234 - Key Fob

MCT-201 WP – Panic Pendant

MCT-302 - Magnetic Contact

NEXT PIR MCW – Supervised PIR

Voice Accessories

VU-20N/U - Dual Voice Message module   MIC-200 - Microphone

Led Keypads

RXN-416 – For 16 Zones

RXN-9 – For 9 Zones

Special Keypads

Wireless Technician Keypad

System Expanders

I/O-8N – 8 Zones, Remote

Programming Modules

LCL-11A – Serial Interface

PRG-22– Fast Programmer

# Appendix – Default system parameters

## System

| Account number (NETsoft Pro): | 0000000000 |
| --- | --- |
| End of service: | 00/00/00 |
| System | CAPTAIN 6 |

## Configuration

| Phone line is connected | + |  | Delayed PGM | - |
| --- | --- | --- | --- | --- |
| Remote test by phone line | - |  | Checking phone line in ON state | - |
| Remote test by radio | - |  | Checking phone line in OFF state | - |
| Auto test by phone line | - |  | Double report | - |
| Auto test by radio | - |  | Report all | - |
| Auto bypass | + |  | Invert Smoke output | - |
| Bypass phone line checking | - |  | Line snapping by 2 rings | - |
| Delayed ON | - |  | Bypass report | - |
| Zones status is displayed in ON | - |  | RX-130 Keypad | - |
| Buzzer keypad with siren | - |  | Request bypass of delayed zones | - |
| Reset per zone | - |  | EOL by two resistors | - |
| Tone dialing | + |  | Listen IN | - |
| Phone line snapping (up to ver. 5.xx) | - |  | Delayed Smoke | - |
| Disable remote downloading | + |  | RX-406 Keypad | - |
| Voice recording unit | - |  | User 8 = Panic | - |
| Zone 6 is arming/disarming key | - |  | Disable Operation if failure | - |
| Non-spring key | - |  |  |  |

## Responses

|  | Siren | Commu. | Buzzer | PGM | Smoke | PSTN reporting | Radio reporting | Beep Siren on Disarming |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Panic | - | - |  | - | - |  |  |  |
| AC fail | - | - | - | - |  |  |  |  |
| Low Battery | - | - | - | - |  |  |  |  |
| Phone line fail | - | - | - | - |  |  |  |  |
| Zone fail | - | - | - | - |  |  |  |  |
| System state | - |  |  | - |  | - | - | - |
| Account in Hex | - |  |  |  |  |  |  |  |

## Zone characteristics

| Feature/No.-> | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Name | ZONE 1 | ZONE 2 | ZONE 3 | ZONE 4 | ZONE 5 | ZONE 6 |
| Sensitivity | 8 | 8 | 8 | 8 | 8 | 8 |
| Bypassed | - | - | - | - | - | - |
| Normally Open | - | - | - | - | - | - |
| 24 Hours | - | - | - | - | - | - |
| Home | + | + | + | + | + | + |
| Delayed | + | - | - | - | - | - |
| Follower | - | + | - | - | - | - |
| EOL loop | - | - | - | - | - | - |
| Responses |  |  |  |  |  |  |
| SIREN output | + | + | + | + | + | + |
| Communication | + | + | + | + | + | + |
| PGM output | - | - | - | - | - | - |
| SMOKE output | - | - | - | - | - | - |
| Partitions |  |  |  |  |  |  |
| Partition 1 | + | + | + | + | + | + |
| Partition 2 | - | - | - | - | - | - |

## Report format and default codes

| Event | Code |  | Event | Code |
| --- | --- | --- | --- | --- |
| Phone format | 0 |  | Test | FF |
| Phone format(Cont) | 0 |  | Panic | FF |
| Phone Account 1 | 0 |  | Fuse fail | FF |
| Phone Account 2 | 0 |  | Fuse restore | FF |
| Zone 1 alarm | FF |  | Arming(General) | FF |
| Zone 2 alarm | FF |  | Disarming(General) | FF |
| Zone 3 alarm | FF |  | Arming(User 1) | FF |
| Zone 4 alarm | FF |  | Disarming(User 1) | FF |
| Zone 5 alarm | FF |  | Arming(User 2) | FF |
| Zone 6 alarm | FF |  | Disarming(User 2) | FF |
| Zone 1 restore | FF |  | Arming(User 3) | FF |
| Zone 2 restore | FF |  | Disarming(User 3) | FF |
| Zone 3 restore | FF |  | Arming(User 4) | FF |
| Zone 4 restore | FF |  | Disarming(User 4) | FF |
| Zone 5 restore | FF |  | Arming(User 5) | FF |
| Zone 6 restore | FF |  | Disarming(User 5) | FF |
| Zone fail | FF |  | Arming(User 6) | FF |
| AC fail | FF |  | Disarming(User 6) | FF |
| AC restore | FF |  | Arming(User 7) | FF |
| Low Battery | FF |  | Disarming(User 7) | FF |
| Battery restore | FF |  | Arming(User 8) | FF |
| Power fail | FF |  | Disarming(User 8) | FF |
| Power restore | FF |  | Siren reset | FF |
| Phone fail | FF |  | Bypass | FF |
| Phone restore | FF |  | Fuse fail | FF |
| Zone 1 alarm | FF |  | Fuse restore | FF |
| Zone 2 alarm | FF |  | Arming(General) | FF |
| Zone 3 alarm | FF |  | Disraming (General) | FF |
| Zone 4 alarm | FF |  | Arming(User 1) | FF |
| Zone 5 alarm | FF |  | Disarming(User 1) | FF |
| Zone 6 alarm | FF |  | Arming(User 2) | FF |
| Zone 1 restore | FF |  | Disarming(User 2) | FF |
| Zone 2 restore | FF |  | Arming(User 3) | FF |
| Zone 3 restore | FF |  | Disarming(User 3) | FF |
| Zone 4 restore | FF |  | Arming(User 4) | FF |
| Zone 5 restore | FF |  | Disarming(User 4) | FF |
| Zone 6 restore | FF |  | Arming(User 5) | FF |
| Zone fail | FF |  | Disarming(User 5) | FF |
| AC fail | FF |  | Arming(User 6) | FF |
| AC restore | FF |  | Disarming(User 6) | FF |
| Low Battery | FF |  | Arming(User 7) | FF |
| Battery restore | FF |  | Disarming(User 7) | FF |
| Power fail | FF |  | Arming(User 8) | FF |
| Power restore | FF |  | Disarming(User 8) | FF |
| Phone fail | FF |  | Siren reset | FF |
| Phone restore | FF |  | Bypass | FF |
| Test | FF |  |  |  |
| Panic | FF |  |  |  |

## Users & partitions

| Part/User-> | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Partition 1 | + | + | + | + | + | + | + | + |
| Partition 2 | + | + | + | + | + | + | + | + |

## Timers

| AC fail | Phone fail | Auto-arming | Auto test | Auto test interval (hrs) | Auto test interval (min) |
| --- | --- | --- | --- | --- | --- |
| 240 | 0 | 00:00 | 00:00 | 0 | 0 |

| Entry delay | Exit delay | SIREN | Waiting for ACK | SMOKE |  |
| --- | --- | --- | --- | --- | --- |
| 20 | 60 | 240 | 20 | 60 |  |

| Siren Tone | 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| Rings number | 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

## Default codes

| Master code: | 5555 |  | User Codes #1 to #8 | ------ |
| --- | --- | --- | --- | --- |
| Technician Code | 1234 |  | Short Code | -- |

# Appendix B - Formats

## Pulse (4-2)

| Format | Rate (pps) | ACK (Hz) | Error Control | A | B |
| --- | --- | --- | --- | --- | --- |
| Ademco Slow | 10 | 1400 | Double Round | 163 | 129 |
|  |  |  | Checksum | 163 | 193 |
|  |  | 2300 | Double Round | 163 | 145 |
|  |  |  | Checksum | 163 | 209 |
| Silent Knight Fast | 14 | 1400 | Double Round | 171 | 129 |
|  |  |  | Checksum | 171 | 193 |
|  |  | 2300 | Double Round | 171 | 145 |
|  |  |  | Checksum | 171 | 209 |
| Franklin | 20 | 1400 | Double Round | 209 | 129 |
|  |  |  | Checksum | 209 | 193 |
| Franklin | 20 | 2300 | Double Round | 209 | 145 |
|  |  |  | Checksum | 209 | 209 |
| Universal High-Speed | 20 | 2300 | Double Round | 83 | 145 |
| Universal High-Speed | 20 | 2300 | Checksum | 83 | 209 |
| Radionics | 40 | 1400 | Double Round | 121 | 129 |
|  |  |  | Checksum | 121 | 193 |
| Radionics | 40 | 2300 | Double Round | 121 | 145 |
|  |  |  | Checksum | 121 | 209 |

## DTMF (4-2)

| Format | ACK (Hz) | Error Control | A | B |
| --- | --- | --- | --- | --- |
| DTMF | 1400 | Double Round | 1 | 130 |
|  |  | Checksum | 1 | 194 |
|  | 2300 | Double Round | 1 | 146 |
|  |  | Checksum | 1 | 210 |
| Contact ID |  |  | 0 | 230 |
| PAF™ | 1400 |  | 0 | 5 |
|  | 2300 |  | 0 | 21 |
| NPAF™ |  |  | Call your dealer |  |
| EPAF™ |  |  |  |  |

# Appendix C – Entering text

Entering text in the CAPTAIN 6 is done similar as in cellular phones: each key has letters and digits assigned to it. Pressing it repeatedly displays the desired letter or number.

Press the / keys to the next character.

Following are the keys and their assigned characters.

| Key | No. of presses |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |
|  | . | , | ? | ! | 1 |  |  |  |  |
|  | A | B | C | 2 |  |  |  |  |  |
|  | D | E | F | 3 |  |  |  |  |  |
|  | G | H | I | 4 |  |  |  |  |  |
|  | J | K | L | 5 |  |  |  |  |  |
|  | M | N | O | 6 |  |  |  |  |  |
|  | P | Q | R | S | 7 |  |  |  |  |
|  | T | U | V | 8 |  |  |  |  |  |
|  | W | X | Y | Z | 9 |  |  |  |  |
|  | Space | Zero |  |  |  |  |  |  |  |
|  | ( | ) | / | * | : | - | + | # |  |
|  | Enable/Disable |  |  |  |  |  |  |  |
|  | Cancel/Return to previous screen without saving |  |  |  |  |  |  |  |
|  | Next character |  |  |  |  |  |  |  |
|  | Previous character |  |  |  |  |  |  |  |
|  | Select/Save |  |  |  |  |  |  |  |

PIMA Electronic Systems LTD

Tel.: +972.3.6506414

Fax: +972.3.5500442

Email: support@pima-alarms.com

Web: http://www.pima-alarms.com

4410061

*4410061*

XX en Rev: L

December 2011

You cannot program a delay for the PGM output and the SMOKE output together. In such a case, the system ignores the SMOKE delay.