New ENCalculator Program Instructions

Creating, Saving and loading files:

- Select File/New from the menu to start a project, you will be prompted for a file name and panel type.

- Select File/Save from the menu to save the current project.

- Select File/Load from menu to load a previously saved project, you will e presented with a list of previously saved file to choose from.

No editing can be done until a new project is started or an existing file opened.

Placing devices:

- Select a device from the tool bar.

- Click anywhere in the form to place a device (not on toolbar or networks).

- To move devices left click and drag them to the new position.

- To square up devices either select Tidy Up from menu Options after placing or select Snap to grid before hand to have all devices placed on a grid.

- To remove devices hold the ctrl key on the keyboard and click the device.

Wiring devices up:

- Left click and drag one device to another to wire them together

- PSUs and networks cannot be dragged to other devices, the devices must be dragged to them.

- Moving devices will move the wiring to keep devices connected.

- To remove wiring hold the alt key on the keyboard and click the device you want unwired.

- Each network has rules which determines how many of each device will be accepted.

Device Status:

Each device has a status light indicating the condition of the device.

- Blue – Device not set up or not on a network.

- Green – Device set up and everything OK.

- Orange – Device low volts in alarm.

- Red – Device low volts.

- Yellow – Device below system grade (ie Grade 2 devices on Grade 3 system)

For the purposes of  testing, low volts is set at 12.5 volts.

Setting up Devices:

- Double click the device to open the set-up window.

- Enter the cable length and impedance.

- Either enter a total current load or click ‘show details’ to add components (PIRs etc)- the loading will be calculated for you.

- If devices are powered by power supplies, both the network cable length and power cable length should be entered.

- The device type can be changed by selecting a new device from the dropdown list. If this would result in too many expanders or keypads on the network, the network link is broken.

Powering Devices:

- Place a PSU on the form.

- Wire the device to be powered to the PSU – the wire must start from the device and go to the PSU

- Devices powered by PSUs will be colour coded to match the PSU

- PSUs can be ‘Cloned’ for use on other networks, a cloned PSU is only a graphical representation in another network. Devices attached to cloned PSUs are in fact being powered by the original PSU.

- Clones are identified by being fully coloured in the colour code of the original.

- Double clicking clones brings up the setup for the original supply.

Reporting

- Click on Report in the menu to display a report of the system structure, status and power usage.

- This report can be saved to file for later printing through Notepad or a similar text editor.