1.3Megapixel HD Cube Network Camera

Features

- 1/3” 1.3Megapixel Aptina CMOS

- H.264 & MJPEG dual-stream encoding

- Max 15fps@1.3M (1280×960)/25/30fps@720P(1280×720)

- DWDR, Day/Night, 2DNR, Auto iris, AWB, AGC, BLC

- Multiple network monitoring: Web viewer, CMS(DSS/PSS) & DMSS

- 3.6mm fixed lens

- White LEDs length 10m

- PIR sensor range 6m

- Micro SD memory (K100A&K100W only), PoE (K100&K100A only), Wi-Fi (K100W only)

Technical Specifications

| Model |  | DH-IPC-K100 | DH-IPC-K100A | DH-IPC-K100W |  |
| --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |
| Image Sensor |  | 1/3” 1.3Megapixel progressive scan CMOS |  |  |  |
| Effective Pixels |  | 1280(H)x960(V) |  |  |  |
| Scanning System |  | Progressive |  |  |  |
| Electronic Shutter Speed |  | Auto/Manual 1/3(4)~1/10000 |  |  |  |
| Min. Illumination |  | 0.01lux/F1.8(color), |  |  |  |
| S/N Ratio |  | >50dB |  |  |  |
| Video Output |  | N/A |  |  |  |
| Camera Features |  |  |  |  |  |
| Max. LEDs Length |  | 10m |  |  |  |
| Day/Night |  | Auto(Electronic) / Color / B/W |  |  |  |
| Backlight Compensation |  | BLC / HLC / DWDR |  |  |  |
| White Balance |  | Auto/Manual |  |  |  |
| Gain Control |  | Auto/Manual |  |  |  |
| Noise Reduction |  | 2D |  |  |  |
| Privacy Masking |  | Up to 4 areas |  |  |  |
| Lens |  |  |  |  |  |
| Focal Length |  | 3.6mm |  |  |  |
| Max Aperture |  | F1.8 |  |  |  |
| Focus Control |  | N/A |  |  |  |
| Angle of View |  | 70° |  |  |  |
| Lens Type |  | Fixed Lens |  |  |  |
| Mount Type |  | Board-in Type |  |  |  |
| Video |  |  |  |  |  |
| Compression |  | H.264/MJPEG |  |  |  |
| Resolution |  | 1.3M(1280x960)/720P(1280x720)/D1/CIF |  |  |  |
| Frame Rate | Main Stream | 1.3M(1~15fps)/720P/D1(1~25/30fps) |  |  |  |
|  | Sub Stream | D1/CIF(1 ~ 25/30fps) |  |  |  |
| Bit Rate |  | H.264: 32K ~ 8096Kbps |  |  |  |
| Audio |  |  |  |  |  |
| Compression |  | N/A | G.711a / G.711u / PCM |  |  |
| Interface |  | N/A | Built-in Mic, Built-in speaker |  |  |
| Network |  |  |  |  |  |
| Ethernet |  | RJ-45 (10/100Base-T) |  |  |  |
| Wi-Fi |  | N/A |  |  | Wi-Fi(802.11b/g) |
| Protocol |  | IPv4/IPv6, HTTP, SSL, TCP/IP, UDP, UPnP, ICMP, IGMP, SNMP, RTSP, RTP, SMTP, NTP, DHCP, DNS, PPPOE, DDNS, FTP, IP Filter, QoS, Bonjour |  |  |  |
| ONVIF |  | ONVIF Profile S |  |  |  |
| Max. User Access |  | 10 users |  |  |  |
| Smart Phone |  | iPhone, iPad, Android, Windows Phone |  |  |  |
| Auxiliary Interface |  |  |  |  |  |
| Memory Slot |  | N/A | Micro SD, Max 64GB |  | N/A |
| RS485 |  | N/A |  |  |  |
| Alarm |  | N/A | 1/1 channel In/Out |  |  |
| PIR Sensor Range |  | Up to 6m |  |  |  |
| General |  |  |  |  |  |
| Power Supply |  | DC12V,PoE(802.3af) |  |  | DC12V |
| Power Consumption |  | <2.6W | <3.4W |  | <6.2W |
| Working Environment |  | -10°C~+50°C, 10%~90% |  |  |  |
| Ingress Protection |  | N/A |  |  |  |
| Vandal Resistance |  | N/A |  |  |  |
| Dimensions(W×D×H) |  | 66.6mm×99.5mm×131.2mm |  |  |  |
| Weight |  | 0.35kg (0.4kg with bracket) |  |  |  |

Dimensions (mm)

Dahua Technology Co., Ltd.

1187 BinAn Road, Binjiang District, Hangzhou, China

Tel: +86-571-87688883

Fax: +86-571-87688815

Email:overseas@dahuatech.com

www.dahuatech.com

*Design and specifications are subject to change without notice.

© 2013 Dahua Technology Co., Ltd.